# Lab 7: SNMP PDU Formats

NET311 - Computer Networks Management

Instructor: Dr. Mostafa Dahshan

## Objectives

- Deeper understanding of SNMPv2 and SNMPv3 protocols.

- Analyzing the formats of different SNMPv2 and SNMPv3 PDUs.

## References

- Using CORE Network Emulator GUI.

- CORE Network Emulator: Video Tutorials.

- CORE Network Emulator: Install Network Services.

- Manpage of snmpd.conf Examples.

- VACM: Net-snmp tutorials.

- VACM (Access Control) configuration.

- Manpage of snmpset.

- Manpage of snmpbulkget.

- netcat - Linuxintro.

## Instructions

- Read the lab instructions.

- Provide question answers and screenshots in the supplied answer sheet.

- After finishing the lab, upload your saved answer sheet to LMS.

# Part 1: Start the Network Environment

1. Start the netlab Linux virtual machine.

2. Login to the netlab Linux virtual machine using login: net311 password: abc.311

3. Run the Terminal emulator

4. To allow access to wireshark, run the following command

sudo dpkg-reconfigure wireshark-common

5. Click Yes.

6. Run the following command

sudo usermod -a -G wireshark $USER

7. Log out.

8. Log in again with the password: abc.311

9. Locate the file NET311-Lab-07.imn included in the lab files. Copy the file to the desktop of the virtual machine.

If you can't use drag-and-drop, you may copy the file contents into a text file using Mousepad. Save the new file under the Desktop folder with the name NET311-Lab-07.imn.

10. Run the CORE Network Emulator

11. From the File menu, open the file NET311-Lab-07.imn from your Desktop folder.

12. Click on the green button to start the session

## Lab sheet 1.1: provide a screenshot showing the running network session.

# Part 2: Analyze SNMPv2 Get-Request and Set-Request PDUs

1. Double-click on server s1 to access its terminal.

2. Review the contents of the file /etc/snmp/snmpd.conf on s1.

cat /etc/snmp/snmpd.conf

## Lab sheet 2.1: provide a screenshot showing the contents of the /etc/snmp/snmpd.conf file on s1.

3. Review the contents of the file /var/lib/snmp/snmpd.conf on s1.

cat /var/lib/snmp/snmpd.conf

4. Right click on the host pc1 to run Wireshark on the link eth0.

5. Enter snmp in the Filter input in Wireshark.

6. Double click on the host pc1 to open its terminal.

7. Send a get-request to host s1 with a single OID (sysUpTime)

snmpget -v2c -c private 10.0.0.12 1.3.6.1.2.1.1.3.0

8. In Wireshark window, double click on the get-request line to inspect its contents

## Lab sheet 2.2: write the values of the PDU fields of the SNMP get-request as indicated in the table.

| Application Header | Version | Community | PDU Type | Request ID | Error Status | Error Index | VarBind 1 name | VarBind 1 value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNMP |  |  |  |  |  |  |  |  |

9. In Wireshark window, double click on the get-response line to inspect its contents

## Lab sheet 2.3: write the values of the PDU fields of the SNMP get-response as indicated in the table.

| Application Header | Version | Community | PDU Type | Request ID | Error Status | Error Index | VarBind 1 name | VarBind 1 value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNMP |  |  |  |  |  |  |  |  |

## Lab sheet 2.4: provide a screenshot showing the PDU fields of the SNMP get-response.

10. Send a get-request to host s1 with a two OIDs (sysUpTime, sysLocation)

snmpget -v2c -c private 10.0.0.12 sysUpTime.0 sysLocation.0

11. In Wireshark window, double click on the get-response line to inspect its contents

## Lab sheet 2.5: write the values of the PDU fields of the SNMP get-response as indicated in the table.

| Application Header | Version | Community | PDU Type | Request ID | Error Status | Error Index | VarBind 1 name | VarBind 1 value | VarBind 2 name | VarBind 2 value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNMP |  |  |  |  |  |  |  |  |  |  |

12. Send a set-request to host s2 with one OID (sysContact.0=Your Name).

snmpset -v2c -c private 10.0.0.10 sysContact.0 s "Mostafa Dahshan"

Note that this requests results in an error, because community private only has read access to the subtree .1.

## Lab sheet 2.6: write the values of the PDU fields of the SNMP get-response as indicated in the table.

| Application Header | Version | Community | PDU Type | Request ID | Error Status | Error Index | VarBind 1 name | VarBind 1 value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNMP |  |  |  |  |  |  |  |  |

# Part 3: Analyze SNMPv2 GetBulk-Request and SNMPv3 Set-Request PDUs

1. On Wireshark, go to Edit > Preferences > Protocols > SNMP. Click on Users Table.

Configure the user user3 with authentication password authpass and privacy password: encrpass for SNMP protocol.

2. Send a send-request to host s2 with one OID (sysContact.0=Your Name), this time using SNMPv3 user user3.

snmpset -v3 -u user3 -a SHA -A authpass -x DES -X encrpass -l authPriv
10.0.0.10 sysContact.0 s "Mostafa Dahshan"

## Lab sheet 3.1: provide a screenshot showing the output of the response to the set-request in Wireshark.

3. Double-click on router r1, then review the ipAddrTable using the following command:

snmptable -c private -v2c localhost ipAddrTable

4. From pc1, send getbulk-request to r1 with 2 non-repetitive objects (sysDescr, sysLocation) and max-repetitions of 3 of the columnar object ipAdEntAddr.

snmpbulkget -v2c -c private -Cn2 -Cr3 10.0.2.1 sysDescr sysLocation ipAdEntAddr

## Lab sheet 3.2: write the values of the PDU fields of the SNMP get-response as indicated in the table.

| Version | Community | PDU Type | Request ID | Non-repeaters | Max-repetitions | VarBind 1 name | VarBind 1 value | … | VarBind 3 name | VarBind 3 value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

5. Send getbulk-request to r1 with 0 non-repetitive objects and max-repetitions of 4 of the two columns ipAdEntAddr and ipAdEntIfIndex.

snmpbulkget -v2c -c private -Cn0 -Cr4 10.0.2.1 ipAdEntAddr ipAdEntIfIndex

## Lab sheet 3.3: provide a screenshot showing the output of the response to the getbulk-request in Wireshark.

# Part 4: Analyze SNMPv2 Trap PDU

1. Double-click on host pc1 to open a terminal, then write the following command to listen for snmp traps.

netcat -u -l 162 | hexdump -C

1. Double-click on host s1 to open the terminal. Then write the following command to trigger a trap.

service snmpd restart

The output on pc1 should look like the following.

The captured packet on Wireshark should look like the following:

## Lab sheet 4.1: write the values of the PDU fields of the last SNMPv2 trap received by pc1.

| Version | PDU Type | Request ID | Error Status | Error Index | VarBind 1 name | VarBind 1 value |
| --- | --- | --- | --- | --- | --- | --- |