# Lab 9: OpenNMS

NET311 - Computer Networks Management

Instructor: Dr. Mostafa Dahshan

Objectives

- Deploy Network Management System Software.

- Install and configure OpenNMS.

- Get hands-on experience with management tasks.

References

- OpenNMS Installation Guide.

- QuickStart.

- Tutorial Discovery.

- Dynamically Configuring DHCP Server Options.

- CNT125 Network Manage Lab 1 of 4 Cisco SNMP.

- CNT125 Network Manage Lab 3 of 4 OpenNMS.

Instructions

- Read the lab instructions.

- Provide question answers and screenshots in the supplied answer sheet.

- After finishing the lab, upload your saved answer sheet to LMS.

# Part 1: Lab Setup

The lab setup required is the same as the lab setup for Lab 05. If you have not performed Lab 05, you must perform Part 1 in Lab 05 before completing this lab.

1. Run PowerShell as Administrator.

2. Type the following command to configure the route to the lab network:

Route add 172.16.0.0 mask 255.240.0.0 172.16.0.2 metric 1

## Lab sheet 1.1: provide a screenshot of the PowerShell screen.

# Part 2: Starting the Network

1. Run GNS3 as an administrator.

2. Open the GNS3 project NET311-Lab-09.gns3.

1. Start all devices

## Lab sheet 2.1: provide a screenshot of the running network.

# Part 3: Install OpenNMS

1. Verify that Java Development Kit is installed. If not, install it using the default options. All installation files are available under the Downloads folder.

2. Run the PostgreSQL installer.

3. When prompted for password for the database superuser (postgres), enter: net311.

4.Make sure the "Stack Builder" option is unchecked, then click Finish.

5. Extract the OpenNMS installer file.

6. Open the INSTALL.txt file in the extracted folder and copy the installation command from there.

7. Open an Administrator command prompt in the extracted folder

8. Paste the installation command

The OpenNMS installer starts.

9. When prompted for the JDK path, click Browse.

10. Select the JDK installation folder.

11. Select the Installation Packages.

12. In the Database Password (Administrator), type: net311.

13. In the Discovery range screen, add the range 172.16.0.1 to 172.16.0.15. We can add more ranges later.

The installation is now finished.

## Lab sheet 3.1: provide a screenshot of the Installation Finished screen of OpenNMS.

# Part 4: Running OpenNMS and Network Discovery.

1. Browse to the OpenNMS installation folder, then run the start.bat file as an Administrator.

2. Run the Firefox web browser and open the address http://localhost:8980. Login with Username: admin and Password: admin.

3. From the admin menu, click Configure OpenNMS.

4. Click on Configure Discovery.

5. Under Include Ranges, click Add New.

6. Add the following address ranges. Set the number of retries to 10 and the timeout to 5000.

7. Click on Save and Restart Discovery.

## Lab sheet 4.1: provide a screenshot showing the Include Ranges after adding all ranges.

8. From the Status menu, click on Events.

9. Click on All events.

## Lab sheet 4.2: provide a screenshot showing part of the events screen.

10. Wait for about 5 minutes for the discovery to update.

11. From the Maps menu, click on Topology.

12. In the Search box, enter:

Iplike:172.16.0.*

Iplike:172.16.2.*

Iplike:172.16.3.*

13. From the View menu in the Topology window, check Automatic Refresh and Link Status.

# Part 5: OpenNMS Network Troubleshooting.

1. In GNS3, double click on R2 to open its terminal.

2. Take down the link between R2 and R1.

conf t

int s0/0

shutdown

3. Check the Status> Events >All events and look for the LinkDown event.

## Lab sheet 5.1: provide a screenshot showing the events screen after the link is taken down.

4. Bring up the link between R2 and R1 again.

no shutdown

5. Check the Status> Events >All events and look for the LinkUp event.

## Lab sheet 5.2: provide a screenshot showing the events screen after the link comes up.