# Lab 10: HP Networks Node Manager

NET311 - Computer Network Management

Instructor: Dr. Mostafa Dahshan

Objectives

- Deploy Network Management System Software.

- Install and configure HP NNMi.

- Troubleshoot network problems.

References

- HP NNMi.

- Configuring Communication Protocol.

Instructions

- Read the lab instructions.

- Provide question answers and screenshots in the supplied answer sheet.

- After finishing the lab, upload your saved answer sheet to LMS.

# Part 1: Lab Setup

The lab setup required is the same as the lab setup for Lab 5. If you have not performed Lab 5, you must perform Part 1 in Lab 5 before completing this lab.

1. Run PowerShell as Administrator.

2. Type the following commands to stop the PostgreSQL service. This is important because this service conflicts with the PostgreSQL database used in HP NNMi.

$s = Get-Service -Name "postgresql-x64-9.5"

Set-Service -InputObject $s -StartupType disabled

Stop-Service -InputObject $s

## Lab sheet 1.1: provide a screenshot of the PowerShell screen.

3. Save your lab sheet, then restart the computer.

4. Run PowerShell as Administrator.

5. Type the following command to configure the route to the lab network:

Route add 172.16.0.0 mask 255.240.0.0 172.16.0.2 metric 1

# Part 2: Starting the Network

1. Run GNS3 as an administrator.

2. Open the GNS3 project NET311-Lab-10-.gns3.

3. Start all devices

## Lab sheet 2.1: provide a screenshot of the running network.

# Part 3: Install HP Network Node Manager

1. Locate the Disc image Software_HP_NNMi_Community_Edition_Windows_TB765-15700.iso under Downloads.

2. Double click on the image to mount it.

3. Locate and open the mounted DVD drive on your computer.

4. Start the setup program.

5. Follow the steps of the setup program. Keep the default settings unless otherwise specified.

6. In the System Account Password field, type net311.

7. Keep the default ports 80 and 443.

8. In the Offical FQDN field, type localhost.

9. Click on Skip Patching.

10. Click on Finish.

## Lab sheet 3.1: provide a screenshot showing the Finish screen of NNM setup.

11. Click No to skip the Quick Start Configuration Wizard.

# Part 4: Configure HP NNM Communication and Discovery

1. Run the Firefox web browser.

2. From the top right corner, open Options. In the General tab click on Make Default.

3. Check the Select All, then click Save.

4. In the Content tab, uncheck the “Block pop-pup windows” option.

5. Locate and run the NNM Console.

6. Sign in with user name system and password net311.

7. In the Network Node Manager window click on Configuration tab, then click on Communication Configuration.

8. Check Enable SNMP Address Rediscovery. Set the SNMP Timeout to 20 and SNMP Retries count to 10.

9. Click the New button under Default SNMPv1/v2 Community Strings.

10. Add the Read Community String public and Ordering 1. Click on Save and Close.

11. In Write Community String, type private and private.

12. Scroll down then change the ICMP Timeout to 20 and ICMP Retries Count to 10.

## Lab sheet 4.1: provide a screenshot showing the Communication Configuration screen.

13. Click on Save and Close.

14. Click on Discovery then click on Discovery Configuration.

15. Open the Auto-Discovery Rules tab, then click on New.

16. Add the Name Lab-10, Ordering 1. Check the options Discover Any SNMP Device, Discover Non-SNMP Devices and Enable Ping Sweep. Then click New.

17. Add the IP Ranges 172.16.0.0/24, 172.16.2.0/24, 172.16.3.0/24. Click Save and Close.

## Lab sheet 4.2: provide a screenshot showing the Auto-Discovery Rule screen.

18. In the Auto-Discovery Rule window, click Save and Close.

19. In the Discovery Configuration window, click Save and Close.

20. Click on Discovery then click on Seeds. Then click on New.

21. Enter Hostname/IP values: 172.16.0.1, 172.16.2.1 and 172.16.3.1. Click Save and Close.

## Lab sheet 4.3: provide a screenshot showing the Seeds screen.

# Part 5: Inspect Network Devices using HP NNM

1. Click on Inventory, then click on Nodes.

## Lab sheet 5.1: provide a screenshot showing the Nodes screen.

2. Double click on R1 and view its information. You can view in a separate window by clicking on .

3. Browse the different tabs on R1.

4. Click on the Interfaces tab of R1. You can view it in a separate window by clicking on .

## Lab sheet 5.2: provide a screenshot showing the Interfaces of ESW1.

5. Close the Interfaces window by clicking on .

6. Under Topology Maps, click on Network Overview.

## Lab sheet 5.3: provide a screenshot showing the Network Overview screen.

# Part 6: Network Troubleshooting using HP NNM

1. Stop one of the nodes in the network, for example PC1.

2. Under Incidents, click on All incidents and locate the Node Down incident for the IP of PC1.

## Lab sheet 6.1: provide a screenshot showing the Incidents screen.

3. In the Topology > Network Overview screen, right-click on R2 then click on Node Access > Ping (from server).

## Lab sheet 6.2: provide a screenshot showing the output of the Ping screen.

4. In the Troubleshooting click on Layer 2 Neighbor View screen, then select the IP of R1.

## Lab sheet 6.3: provide a screenshot showing the output of the Layer 2 Neighbor View screen.

5. Right-click on R1, then click on MIB Information -> MIB Browser.

6. Click on SNMP Walk.

## Lab sheet 6.4: provide a screenshot showing the output of SNMP Walk in the MIB Browser.